LINKCRIB

Camera & Security Compatibility Guide

What works today, what could work next, and how LinkCrib avoids unnecessary vendor lock-in

| CURRENT BUILD Alpha v1.18 | PROVEN CAMERA PLATFORM Reolink + Home Hub | DIRECTION Local-first and extensible |
| --- | --- | --- |

# The short answer

Today, Reolink is the only camera ecosystem implemented and tested end to end in LinkCrib. LinkCrib is not intended to remain locked to one brand. Future integrations should favor products with documented local APIs or open interoperability standards, while cloud-only services remain portal links unless a safe, supported connection becomes available.

# Compatibility status

| STATUS | PLATFORM | LINKCRIB POSITION | WHAT THAT MEANS |
| --- | --- | --- | --- |
| Tested | Reolink | Current recommended camera system | Inventory and snapshots are working with the Reolink Home Hub. |
| Strong candidate | UniFi Protect | Best next full security-platform integration | Official API covers cameras, snapshots, sensors, sirens, relays, and alarm hardware. |
| Planned direction | ONVIF Profile T | Best route to multi-brand camera support | A generic connector could support verified conformant models without writing a separate adapter for every brand. |
| Possible | RTSP cameras | Basic stream or snapshot path | Useful for video access, but usually provides less inventory, health, event, and sensor data than ONVIF. |
| Portal only | Ring, ADT Control, Alarm.com and similar | No direct account integration planned today | Users can reach the provider’s portal without LinkCrib repeatedly authenticating against a restricted cloud account. |

# Recommended alternatives to Reolink

## 1. UniFi Protect

Why it fits LinkCrib: UniFi publishes an official Protect API with camera inventory, snapshots, connection state, sensors, sirens, alarm hubs, lights, relays, and event updates. Protect also stores recordings and performs supported analytics locally on its console or NVR.

Best suited to users who want a coordinated camera, sensor, and local-recording ecosystem.

Would require a dedicated LinkCrib UniFi Protect connector and user-supplied local API credentials.

Not yet implemented or certified in the current LinkCrib build.

## 2. Verified ONVIF cameras

Why it matters: ONVIF defines interoperable interfaces for IP-based security products. A LinkCrib ONVIF adapter could discover cameras, read capabilities, request snapshots, and report device state across multiple manufacturers.

Favor ONVIF Profile T for new purchases; it provides a more modern streaming and authentication direction.

Verify the exact model and firmware in ONVIF’s official conformant-products database before purchasing.

Potential brands include selected models from Axis, Amcrest, Hanwha, TP-Link VIGI, Hikvision, Dahua, and others—but the brand name alone does not guarantee compatibility.

## 3. Direct RTSP cameras

Appropriate for basic viewing: RTSP can expose a local video stream that LinkCrib could display or convert into periodic snapshots. It should be treated as a limited fallback because RTSP alone normally lacks friendly inventory, rich health information, events, and security-sensor state.

# What buyers should look for

Local access that continues working when the internet connection is unavailable.

ONVIF Profile T conformance or a documented local REST, WebSocket, MQTT, or similar API.

A documented snapshot endpoint and stable camera identifiers.

Local accounts or scoped API keys instead of sharing the owner’s primary cloud password.

No mandatory subscription for basic local viewing and status reporting.

Clear firmware support, security updates, and a published privacy policy.

# Security devices beyond cameras

LinkCrib could eventually report and automate additional locally accessible devices, including:

Door and window contact sensors

Motion and occupancy sensors

Glass-break sensors

Water-leak, smoke, and environmental sensors

Sirens, security lights, relays, and garage-door state

Smart-lock state, subject to strict authorization and safety controls

## Preferred integration paths

| TECHNOLOGY | GOOD USE | LINKCRIB APPROACH |
| --- | --- | --- |
| Matter | Cross-brand household sensors and locks | Develop after LinkCrib’s shared device model and security permissions are mature. |
| Zigbee / Z-Wave | Battery sensors and established home-security devices | Use a local coordinator or controller; avoid brand-cloud dependence. |
| MQTT / local API | DIY, professional, and hub-based status reporting | Use scoped credentials and read-only monitoring first. |
| ONVIF | Video, metadata, and selected access-control systems | Begin with camera discovery, state, and snapshots before control features. |

# LinkCrib compatibility labels

Tested: Implemented and validated on real LinkCrib hardware.

Compatible: Implemented and expected to work with documented requirements, but not part of the primary test system.

Experimental: Available for evaluation with limitations clearly identified.

Portal only: Provides safe access to the vendor’s own service without LinkCrib handling the account session.

Unsupported: No safe or maintainable integration path is currently offered.

# Recommended roadmap

Continue treating Reolink as the proven recommended camera option.

Build a generic ONVIF Profile T camera connector for verified conformant devices.

Add UniFi Protect as the second fully supported camera and security ecosystem.

Introduce read-only local sensor reporting before any security control or arming features.

Keep restricted cloud systems portal-only unless their vendors provide safe, documented APIs.

# Important purchasing note

Do not purchase a device solely because its product page says “ONVIF compatible.” Confirm the exact model and installed firmware in ONVIF’s official database, and check LinkCrib’s published compatibility status. Features such as PTZ, audio, event metadata, and snapshots can vary by model. This Alpha v1.18 guide describes direction, not a promise that an untested product will work.

# Authoritative references

ONVIF Profiles

ONVIF Conformance FAQ

ONVIF Profile S deprecation guidance

Official UniFi Protect API

How UniFi Protect protects and stores data